# ECSE211 Lab 2 – Inputs, Outputs, Testing

Report Due: Monday, September 29th, 2025, 11:59PM

# Objectives

By now, you should have finished Lab 1 and have selected an idea for your mini project. With the system level design from the previous lab, you should also have a clear idea of the inputs (sensors/motors) and outputs (sound) involved in your design. The objectives of this lab are to:

- Give you hands-on experience in working with the DPM kit, your main design material.

- Help you understand the characteristics of the different sensors you have available as input devices.

- Help you understand how to design and conduct tests.

You can compare this lab to the Research & Development phase of an engineering project; after generating requirements and preliminary designs, tests must be conducted to evaluate the feasibility of the design.

# Grade Breakdown

- Demonstration to the TA: 25%

- Code quality and functionality: 5%

- Lab Report: 70%

This document contains instructions to follow to complete the lab, along with the content that should be in the report for each section. Everything mentioned in the “To include in report” subsections of this document must be presented in the lab report in the same structure as below.

# Lab Setup

## Materials

You will need the following items from the DPM kit:

- The BrickPi

- A touch sensor

- An ultrasonic sensor

- A color sensor

- Sensor cables

- Speaker or headphones with an AUX jack cable

- Set of 6 colored cubes (if you don’t have any, ask a TA to give them to you)

You will also need a computer for the programming component of the lab. Alternatively, you may use the monitors available in the DPM lab and write your code directly on the BrickPi.

## Team Organization

- Each team should complete the lab to test all three types of sensors as instructed below.

- Each member should thoroughly keep track of the time they spend on different tasks in the lab.

## Starter Code

Download the starter code from myCourses and read the instructions on how to use it. For this lab, you will need the 3 following files:

- speaker_button.py

- collect_us_sensor_data.py

- collect_color_sensor_data.py

Some of them will require modifications. You are allowed to add your own modules, classes, and functions, but you cannot change the submission structure (i.e., you must keep the original structure of the code).

To test your starter code setup, connect the speaker’s 3.5mm AUX jack to the audio port of the BrickPi. Run the speaker_button.py script (from the starter code) and confirm that a beeping sound can be heard. Keep the speaker connected to the BrickPi for the remainder of the lab.

## To include in report

Your lab report should have a section titled “Section 1: Lab Setup” containing the following:

    - List the team members along with the portions of the lab each person worked on.

Continued next page.

# Touch Sensor Testing

### Task

Implement a function that plays a sound when the button (touch sensor) is pressed. In the file speaker_button.py, implement the function play_sound_on_button_press() that will sample the touch sensor and play a sound when it is pressed.

The code should contain:

- A while loop that continuously samples the touch sensor.

- An if statement that checks for the touch sensor status.

Hint: You can get the status of the touch sensor by calling “TOUCH_SENSOR.is_pressed()”, which should return a boolean.

- A call to play_sound() if conditions are met.

## To include in report

Your lab report should have a section titled “Section 2: Touch Sensor Testing” containing the following:

- Report qualitative results on the efficiency of the touch sensor based on your test rez`zsults.

- Discuss whether the touch sensor will be suitable for the remainder of the project based on your findings.

- What are the capabilities and limitations of the touch sensor in the DPM kit?

Continued next page.

# Ultrasonic Sensor Testing

### Task

For this test, we will move the robot along a parallel surface and observe the data collected by the US sensor. Keep in mind that, assuming you are moving the robot at a perfectly constant distance from the surface, the value measured should be constant (this probably won’t happen… think about why).

Follow these steps to conduct the test:

- Connect the Touch Sensor to port S1 and the Ultrasonic Sensor to port S2.

  - Ensure the Ultrasonic Sensor is firmly attached to the robot.

  - The Touch Sensor may be held freely in one hand while you perform this test.

  - You may use either battery or wired connection to power the robot.

- Run the function collect_continuous_us_data() on the robot (project/collect_us_sensor_data.py)

  - Place the robot along a surface that is parallel to another e.g., in a hallway or a box.

  - Press the attached Touch Sensor once, then slide the robot along the surface while trying to keep the distance to the surface constant.

  - Keep sliding the robot for a distance of at least 60cm. Then, press the touch sensor again to stop saving data to the file (see figure 1; the blue line should be at least 60cm long).

Figure 1: Bird’s eye view of the robot’s trajectory for the US sampling test

- The data will be saved automatically in the lab2/data_analysis/us_sensor.csv file in your project.

- Generate the graph resulting from the .csv file using the guidelines provided in Appendix I.

You will need the .csv file and the generated graphs for the report – keep them handy!

## To include in report

    - Test Results

Perform the test procedure described in section 3.1 of this handout for distances of 10cm, 20cm and 30cm and present a table containing the following data:

      - Minimum value recorded by the US sensor.

      - Maximum value recorded by the US sensor.

      - Mean of the recorded values.

      - Standard deviation of the recorded values.

In addition, provide the graphs generated by the graphing tool.

Your table should contain three rows of data and you will have three graphs to present.

Your lab report should have a section titled “Section 3: Ultrasonic Sensor Testing” containing the following

information:

- Provide a table as described above to report quantitative results on the efficiency of the ultrasonic sensor based on the results of the test.

- Discuss any performance variations you observed during the tests. Why do you think they occurred? What steps would you take to address them if they persisted?

- Evaluate whether the ultrasonic sensor will be suitable for the remainder of the project based on your findings.

## Discussion

Additionally, answer the following discussion questions:

- What is the sampling frequency of the US sensor used in this lab?

- From the graphs generated in the Test Data, how would you describe the accuracy of the sensor? Clearly explain what makes you draw this conclusion.

- Does the amount of noise change as the distance to the wall changes? If so, explain how.

- Can a noisy sensor impact the performance of a design? Why or why not?

Continued next page.

# Color Sensor Testing

### Task

In this part, you will implement a discrete color sampler using the touch sensor. In the collect_color_sensor_data.py file, complete the collect_color_sensor_data() function such that when the touch sensor is pressed once, the program reads and prints the RGB values from the color sensor once, then writes the values to a file. The number of measurements should be equal to the number of touch sensor presses.

The code should contain:

- A statement to open the output file (refer to collect_us_sensor_data.py for an example).

- A while loop that continuously samples the color sensor.

- An if statement that checks for the touch sensor status.

- An action to write the color sensor reading to the output file when conditions are met. Then, follow these steps to conduct the test:

- Pick a colored foam cube and position it very close to the color sensor input (about 5mm from it).

- Run the script collect_color_sensor_data.py

- Press the touch sensor and make sure that it collects only one RGB Data point.

- Use the touch sensor to collect at least 10 data points from the colored cube.

- The data should be saved automatically in the lab2/data_analysis/color_sensor.csv file in your project. Verify that this is the case.

- Generate the graph resulting from the .csv file using the guidelines provided in Appendix I. You should obtain 1 graph for each cube sampled, and each graph should contain 3 bell curves.

- Repeat steps 1-6 with 2 other foam cubes of different colors.

The distribution generated by the graphing tool represents the profile of a real-world color. There are 3 Gaussian Bell Curves, one for each color Red, Green, and Blue. If an RGB value collected from the color sensor has its R value, G value, and B value fit underneath or on each of these 3 bell curves, then the collected RGB value matches this color.

Note: At the end of this section, you will know the “standard” RGB values of 3 different colored cubes. When you sample the RGB values of a cube of unknown color, you can try to match the sample values with the colors you know the values for. Then, if you find a color whose set of values is close enough to your unknown color, you can assume that the block is of that color.

## To include in report

    - Test Results

Perform the test procedure described in section 4.1 of this handout for three different colors (including at least red) and present a table containing the following data:

      - Mean and standard deviation of the normalized R values recorded by the sensor.

      - Mean and standard deviation of the normalized G values recorded by the sensor.

      - Mean and standard deviation of the normalized B values recorded by the sensor.

In addition, provide the graphs generated by the graphing tool. Your table should contain three rows of data and you will have three graphs to present.

Your lab report should have a section titled “Section 4: Color Sensor Testing” containing the following:

- Provide a table as described above to report quantitative results on the efficiency of the color sensor based on your test results.

- Discuss any performance variations you observed during testing. Why do you think they occurred? What steps would you take to address them if they persisted?

- Evaluate whether the color sensor will be suitable for the remainder of the project based on your findings.

## Test Procedure

Write up a full test procedure for the color sensor test by filling out the Test Procedure Template (check MyCourses – It is listed with this document).

## Classification Algorithm

Using the results obtained from the color sensor testing, design a color classification algorithm to determine the color of a cube based on its RGB values. In other words, give a detailed description of how you would detect and differentiate colors using the color sensor. Your answer to this question should be a list of steps your algorithm would have. You do not have to write the Python code for the function; simply tell us how you would approach the problem.

## Discussion

- Explain how you normalized the RGB values in your test data.

- Look at the graphs generated by the graphing tool. What does the position of the Gaussian curves represent for the RGB value? Explain why the graph generated with a red cube has curves so far from each other.

- From the graphs generated in the Test Data, how would you describe the accuracy of the sensor? Clearly explain what makes you draw this conclusion.

- Are there colors for which the sensor was particularly accurate? State which ones and formulate a hypothesis as to why.

- Are there colors for which the sensor was not accurate at all? State which ones and formulate a hypothesis as to why.

Continued next page.

# Design Iteration

## To include in report

Your lab report should have a section titled “Section 5: Design Iteration” with the following:

- Initial Design Description:

  - Briefly describe the system you had selected in Lab 1.

  - Now that you have more concrete ideas (e.g., quantitative measurements) about the capabilities and limitations of the input devices, re-write the specifications for each component of your system.

- Feasibility: Referring to the results of the tests you’ve conducted, and the specifications you’ve outlined, describe whether your initial idea is feasible as it was originally envisioned in Lab 1.

- Design Selection:

  - Has your decision changed to modify your initial idea, or to follow the new design idea?

  - How did the tests help make the decision for your group?

  - Provide a conclusion on which idea you will pursue to implement in the next two weeks.

For each question in this section, provide answers in the form of a paragraph (no bullet-point lists). This section should be maximum 1 page long, excluding figures.

# Team Reflection

## To include in report

Your lab report should have a section titled “Section 6: Team Reflection” with the following:

- Timesheet: Provide a summary of how long it took each member of the group to do different tasks involved in this lab. This should be presented as a table or a figure. (In this section, the organization of the table and the extent of the information presented is evaluated).

- Time Management: Describe whether members of the group are satisfied with the time it took for each type of task to be completed by the group. If not, outline how the team will do things differently in the subsequent labs.

# Code submission

You should submit the entire source code folder in a .zip file on myCourses. You will be evaluated on code functionality, style, and documentation.

# Demonstration to the TA

You will need to demonstrate your work to a TA during one of your team member’s lab slots. Teams of 6 should have 5+ students present, while teams of 5 should have 4+ students present during the demo.

The demonstration must be completed by Friday, September 26th, to accommodate for your team’s schedules. However, the lab code and report must be submitted by the deadline, regardless of your demo date.

During the demo, your TA will ask you questions about your testing procedures and group work together. And each team needs to demonstrate the following:

- Demonstrate that the speaker plays a sound when you press the attached touch sensor.

- Demonstrate your testing procedure for collecting Ultrasonic Sensor data.

- Demonstrate your testing procedure for collecting Color Sensor data.

- Show your TA the data that you have collected and answer questions on what it means.

Appendix I: Lab Report Grade Breakdown

| Section | Subsection (if applicable) | Points |
| --- | --- | --- |
| 1 (Lab Setup) | Team Organization | 3 |
| 2 (Touch Sensor) | Test Results | 5 |
| 3 (US Sensor) | Test Results | 8 |
|  | Discussion | 8 |
| 4 (Color Sensor) | Test Results | 8 |
|  | Test Procedure | 14 |
|  | Classification algorithm | 5 |
|  | Discussion | 8 |
| 5 (Design Iteration) |  | 6 |
| 6 (Team Reflection) |  | 5 |
| Total |  | 70 |